Dependencies and where to download them

- Downlaod SimNIBS and any additional dependencies (depending on version) from https://simnibs.github.io/simnibs/build/html/installation/simnibs_installer.html

  - For SimNIBS 4, also download FreeSurfer from https://surfer.nmr.mgh.harvard.edu/fswiki/DownloadAndInstall

  - For Windows users, you can download and use FreeSurfer through the Linux subsystem by following the instructions here https://surfer.nmr.mgh.harvard.edu/fswiki/FS7_wsl

- Download the SimNIBS example dataset from https://simnibs.github.io/simnibs/build/html/dataset.html

- Download or clone TAP from https://github.com/moritzdannhauer/TAP

Checking the TAP installation

- Copy M1_mask_TMS_target.nii.gz from TAP/subjects/Ernie_headreco into “ernie”

- Open “run_subj.m”

- Change the filepaths between “%%%%% CHANGE TAP PARAMETER HERE: Begin %%%%%” and “%%%%% CHANGE TAP PARAMETER HERE: End %%%%%” as needed

  - “fsl_path” is the path to the FSL folder (ignore this for SimNIBS 4)

  - “simnibs_folder” is the path to the SimNIBS folder

  - “root_folder” is where you downloaded TAP

  - “freesurfer_matlab_folder” is the matlab folder within the freesurfer installation

  - “Coil_models_are_here” is SimNIBS folder where the coil models are stored. It’s usually in resources

  - Insert “addpath([simnibs_])”

  - Change “addpath(char[simnibs_folder sep matlab])” to “addpath(<pathToMATLAB>)”

  - Change the line starting with coil_name to “coil_name=MagVenture_Cool-B65.ccd”

- Change “subjects/Ernie_charm” or the folder corresponding to the appropriate pipeline to “subjects/ernie” and copy the “ernie” folder from the example dataset to the TAP folder

- Run “run_subj.m”

Checking the FreeSurfer integration with SimNIBS 4

- In the Linux subsystem / Terminal, run “recon-all -subjid ernie -all -i <pathToErnieT1> -T2 <pathToErnieT2>”

  - Replace the “<>” and its enclosed information with the appropriate filepaths

  - This should generate a “fs_ernie” folder in the “SUBJECTS_DIR” you specified during the FreeSurfer Installation

  - If you get a permission denied error, add “export SUBJECTS_DIR=<acceessable folder>” to the same file you added “source $FREESURFER_HOME/SetUpFreeSurfer.sh” while setting up freesurfer

- Follow the instructions at https://simnibs.github.io/simnibs/build/html/tutorial/head_meshing.html to create the head model, but replace the “charm ernie org/ernie_T1.nii.gz org/ernie_T2.nii.gz” command with “charm_tms ernie org/ernie_T1.nii.gz org/ernie_T2.nii.gz --fs-dir <pathTofs_ernie>”